**This needs to be setup before running the python codes.

Logistics

For the setup, prepare the following hardware:

1. Raspberry Pi (2011 Model B)

2. 8GB SD Card

3. 5V Power Adapter for Raspberry Pi

4. Ethernet Cable

5. Router to Access to Network

6. Female/Female Jumper Wires (4X)

7. Adafruit PN532 Breakout Board

8. Jumpers [Plugs] (2X)

9. Headers (2X 3-pin & 1X 6-pin)

10. Solder & Soldering Iron

11. NFC Type 2 Tags/MiFare Ultralight Tags

12. Laptop/PC

13. *Speaker

14. *LED(s)

15. *Resistor(s)

16. *Wire-wrapping wires (Or insulated copper wires)

17. *Breadboard

18. *Power Bank

19. *Items to Make Casing (e.g. Scissor, cutter, tapes, glue, corrugated board, etc.)

*Optional items

Equipment list diagram:

Setting Up the PN532 Breakout Board

To begin, solder in the header to the breakout board. You'll need two small 3-pin pieces of header. One 8-pin piece is also optional. You can break these off a large piece.

Solder the 2X 3-pin pieces to the SEL0 and SEL1 pads. These are interface selectors for the chip. Depending on how the jumpers are inserted the chip will talk in UART, I2C or SPI. For our purpose we will be using UART Serial.

(Optional) Also solder the 8-pin piece to the FTDICABLE pads. These will be used to interface with the raspberry pi. (Edit: If not needed in the final prototype, do not solder the 8-pin piece on as it might pose to be an obstruction when making the final prototype)

PN532 Board with headers soldered to selectors

*Depending on the preferred positioning of the breakout board in the final prototype, the longer pins can be soldered to either face the bottom or the top of the board.

(Author’s note: 8-pin was unnecessary in prototype, and longer pins would have fit better at the underside of the board as opposed to Figure 3.)

For more info, see https://learn.adafruit.com/adafruit-pn532-rfid-nfc/breakout-wiring

Setting Up Raspberry Pi

Installing Raspbian OS

First, the Raspbian Jessie OS has to be written into the SD Card. To do so, download the Raspbian Jessie image from the following URL:

https://www.raspberrypi.org/downloads/raspbian/

Under Raspbian Jessie, click “Download ZIP” and save the zip file into your laptop. Extract the image file.

Insert the SD card into your SD card reader and check which drive letter was assigned (e.g. G:).

Download the Win32DiskImager utility (it is also a zip file) from the following URL:

http://sourceforge.net/projects/win32diskimager/

Extract the executable from the zip file and run the Win32DiskImager utility; you may need to run the utility as administrator. Right-click on the file, and select Run as administrator.

Select the Raspbian Jessie image file you extracted above.

Select the drive letter of the SD card in the device box. Be careful to select the correct drive; if you get the wrong one you can destroy your data on the computer's hard disk!

Click Write and wait for the write to complete.

Exit the imager and eject the SD card.

*Also, Win32DiskImager can be used to back-up the SD card. For more information refer to the following URL:

http://lifehacker.com/how-to-clone-your-raspberry-pi-sd-card-for-super-easy-r-1261113524

SSH into Raspi

Connect to Raspi via Putty (you need to find out the IP address of the Raspi, 1 way is to key in the default gateway IP to see devices connected to your router).

Freeing UART On Pi

To free up UART for our purpose, enter the following command into the command prompt:

$ sudo raspi-config

From the main menu, select “9 Advanced Options” >> “A8 Serial”, then select 'No' to disable shell and kernel messages via UART.

From the command prompt enter the following commands to reboot your Raspberry Pi board, freeing UART up in the process:

$ sudo reboot

Reconnect by restarting PuTTY application and reconnecting over SSH.

For more info, see: https://learn.adafruit.com/adafruit-nfc-rfid-on-raspberry-pi/freeing-uart-on-the-pi

Hooking PN532 Up With Raspberry Pi

First, attach the jumpers (plugs) on the right 2 pins of selectors, SEL0 and SEL1, to select UART connection.

To connect the Raspberry Pi to the PN532 breakout board via the FTDICABLE, use the 4 F/F jumper wires to connect the 5V, GND, TXD and RXD ports together.

IO Ports of Raspberry Pi

Hooking Up Raspberry Pi with PN532 Breakout Board

Optional Enhancements

Sound Effects

You will additionally need a speaker with a 3.5mm Jack to connect to the Raspberry Pi. This will work with the mpg123 library

Lighting Effects

For this, you will additionally need LED(s), resistor(s) and a breadboard (unless you are directly connecting the LED(s) and resistor(s) to the Pi)

To add lighting effects upon reading a tag, the GPIO Pins on the Raspberry Pi can be used generate power to light up LEDs.

Pin Configuration on a Raspberry Pi Model B ()

When we use a GPIO pin as an output, each pin can turn on or off, or go HIGH or LOW in computing terms. When the pin is HIGH it outputs 3.3 volts (3v3); when the pin is LOW it is off.

For this example, we will connect the positive end of the LED to the GPIO (e.g. #25) and the negative end to Ground.

This GPIO will work with the RPi.GPIO library installed via pip

Setup of LED and Resistor on a Breadboard

See more at (https://www.raspberrypi.org/documentation/usage/gpio/)

Increasing the Area of Detection for NFC Antenna

Since the NFC Antenna is basically a solenoid/inductor, the area of detection of NFC tags can be increased by overlaying the antenna with an enclosed coil of copper wires. Make sure to have adequate number of loops in the coil of wire (while trying to create a large coil) to increase the strength of detection.

make a big circle while looping the enclosed wire many times.